Assignment 3 - Electricity Questions

- A current of 0.08A flows through a bulb when a p.d. of 6V is put across it.

Calculate:

  - The resistance of the bulb

  - The power of the bulb

  - The amount of charge which flows through it every minute.

- 3 10k resistors are arranged in series in a circuit.

a) Calculate their combined resistance.

b) What would their combined resistance be if arranged in parallel?

c) What would the conductance of a parallel combination be?

- A 70W electric motor draws a current of 6.5A from a power supply of e.m.f. 12V.

Calculate :

  - The voltage across the terminals of the power supply

  - The resistance of the motor

  - The internal resistance of the power supply

- A light sensor with an internal resistance of 500 is known to produce an e.m.f. of 300V at a certain light intensity. To measure this voltage a students uses a voltmeter with a resistance of 50k.

  - What current will the voltmeter draw from the sensor?

  - What will the voltmeter read?

  - Suggest how a reading closer to the e.m.f. could be obtained.

Another student suggests that lots of these light sensors could be joined together to produce enough voltage to run a small motor.

  - How many sensors would be needed to provide an e.m.f. of 6V?

  - What would the internal resistance of such a combination be?

  - Explain why the other student’s idea is impractical.